Protoss-PE11

RS485 to Ethernet

User Manual

V 1.0

Overview of Characteristic

- Cortex-M3 MCU with 2MB Flash and 128KB SRAM

- Use FreeRTOS Operation System

- Support TCP/UDP/MQTT/HTTP/WebSocket Protocol

- Support Modbus TCP to RTU, Modbus Master Function

- Support RS485 To 10/100M Ethernet Conversion, Serial Speed Up to 460800 bps

- Support 10/100M Ethernet Auto-Negotiation

- Support Webpage Easy Configuration or PC IOTService Tool

- Support Security Protocol Such As AES/DES3

- Support Heartbeat and Resister Packet Function

- Support Webpage OTA Wireless Upgrade

- Support Industrial Temperature: -40 to +85˚ C

- Multiple Type of Different Power Input：

  - Protoss-PE11-H：100～240VAC@50～60Hz

  - Protoss-PE11-M：9～48VDC@1A

- Size: 97.60 x 64.95 x 27.50 mm (L x W x H) ，C45 rail installation

- Table Of Contents Table Of Contents

Table Of Contents Table Of Contents	3

List Of Figures	4

List Of Tables	5

History	5

1.	Product Overview	6

1.1.	General Description	6

1.2.	Device Paremeters	6

1.3.	Key Application	7

2.	Hardware Introduction	8

2.1.	Protoss-PE11 Pins Definition	9

2.2.	RS485 Interface	10

2.3.	RJ45 Interface	10

2.4.	Mechanical Size	11

2.5.	Product Installation	13

2.6.	Order Information	13

3.	Function DESCRIPTION	14

Appendix A: Contact Information	15

List Of Figures

Figure 1.	Protoss-PE11 Appearance	8

Figure 2.	Protoss-PE11 Interface	9

Figure 3.	RJ45 Pin Defination	10

Figure 4.	Protoss-PE11 Mechanical Dimension	12

Figure 5.	Product Installation	13

Figure 6.	Protoss-PE11 Product Order Information	13

List Of Tables

Table1.	Protoss-PE11 Technical Specifications	6

Table2.	Protoss-PE11-H Interface Definition	9

Table3.	Protoss-PE11-M Interface Definition	10

Table4.	RJ45 Interface	11

History

Ed. V1.0   02-10-2020 		First Version

- Product Overview

  - General Description

The Protoss-PE11 provides a RS485 interface to TCP/IP data transfer product. The Protoss-PE11 integrate TCP/IP controller, memory, 10/100M Ethernet transceiver, RS485 and integrates a fully developed TCP/IP network stack and FreeRTOS OS. Protoss-PE11 also includes an embedded web server used to configure device.

The Protoss-PE11 using highly integrated hardware and software platform, it has been optimized for all kinds of applications in the industrial control, smart grid, personal medical application and remote control that have lower data rates, and transmit or receive data on an infrequent basis.

  - Device Paremeters

Protoss-PE11 Technical Specifications

| Item | Parameters |
| --- | --- |
| System Information |  |
| Processor/Frequency | Cortex-M3/96MHz |
| Flash/SDRAM | 2MB/128KB |
| Operating System | FreeRTOS |
| Ethernet Port |  |
| Port Number | 1 |
| Interface Standard | 10/100 Base-T Auto-Negotiation |
| Transformer | Integrated |
| Network Protocol | IP，TCP，UDP，DHCP，DNS，HTTP Server/Client，ARP, AutoIP, ICMP， Telnet, NTP, Modbus TCP |
| Security Protocol | AES 128Bit DES3 |
| Serial Port |  |
| Port Number | 1 RS485 |
| Data Bits | 5,6,7,8 |
| Stop Bit | 1,2 |
| Check Bit | None, Even, Odd |
| Baud Rate | TTL: 600 bps~460800 bps |
| Flow Control | No Flow Control Software Xon/ Xoff flow control |
| Software |  |
| Web Pages | Http Web Configuration Customization of HTTP Web Pages |
| Configuration | Web CLI XML import Telnet IOTService PC Software UART Fast Config |
| Firmware Upgrade | Webpage, IOTService Tools |
| Basic Parameter |  |
| Size | 97.60mm x 64.95mm x 27.50mm |
| Operating Temp. | -40 ~ 85°C |
| Storage Temp. | -45 ~ 105°C, 5 ~ 95% RH（no condensation） |
| Input Voltage | Protoss-PE11-H：100～240VAC@50～60Hz Protoss-PE11-M：9～48VDC@1A |
| Working Current | ~100mA |
| Power | <400mW |

  - Key Application

The Protoss-PE11 device connects serial device to Ethernet networks using the TCP/IP protocol:

- Remote equipment monitoring

- Asset tracking and telemetry

- Security Application

- Industrial sensors and controls

- Medical devices

- ATM machines

- Data collection devices

- Universal Power Supply (UPS) management units

- Telecommunications equipment

- Data display devices

- Handheld instruments

- Modems

- Time/attendance clocks and terminals

- Hardware Introduction

The Protoss-PE11 unit is a complete solution for serial port device connecting to network. This powerful device supports a 10/100BASE-T Ethernet connection, a reliable and proven operating system stored in flash memory, an embedded web server, a full TCP/IP protocol stack, and standards-based (AES) encryption.

Through Ethernet cable connect router with Protoss-PE11 serial server for data transfer, which makes the data transformation very simple.

Protoss-PE11 Appearance

  - Protoss-PE11 Pins Definition

Protoss-PE11 Interface

Protoss-PE11-H Interface Definition

| Pin | Description | Net Name | Signal Type | Comment |
| --- | --- | --- | --- | --- |
| 1 | AC Power Input | L | Power | 100～240VAC Input |
| 2 | AC Power Input | N | Power |  |
| 5 |  | RS485_B- | IO | RS485 B- |
| 6 |  | RS485_A+ | IO | RS485 A+ |
| 7 | Signal GND | GND | Power | Used for RS485 GND, usually leave it unconnected |
| RJ45 | Ethernet | RJ45 | I/O |  |
| Reload | Restore to factory setting button | Reload | I | Press down for more than 3 seconds and loose to restore factory setting. |
| Reset | Reset button | Reset | I | Hardware reset button |
| Net | Network status LED | Net | O | On: Ethernet connection is OK Off: No Ethernet connection |
| Active | UART Data Transfer | Active | O | Off: No data transfer 0.3s Off -> 0.9s On: UART TX Output 0.3s Off -> 0.3s On: UART RX Receive On: UART bidirection. |
| Power | Power LED | Power | O | On: Power input OK Off: Power input NG. |
| Link | Server connection LED | Link | O | On: netp Socket connection OK. Off: no netp Socket connection. |

Protoss-PE11-M Interface Definition

| Pin | Description | Net Name | Signal Type | Comment |
| --- | --- | --- | --- | --- |
| 1 | DC Power Input | VCC+ | Power | 9～48VDC@1A Input |
| 2 | DC Power Input | GND- | Power |  |
| 5 |  | RS485_B- | IO | RS485 B- |
| 6 |  | RS485_A+ | IO | RS485 A+ |
| 7 | Signal GND | GND | Power | Used for RS485 GND, usually leave it unconnected |
| RJ45 | Ethernet | RJ45 | I/O |  |
| Reload | Restore to factory setting button | Reload | I | Press down for more than 3 seconds and loose to restore factory setting. |
| Reset | Reset button | Reset | I | Hardware reset button |
| Net | Network status LED | Net | O | On: Ethernet connection is OK Off: No Ethernet connection |
| Active | UART Data Transfer | Active | O | Off: No data transfer 0.3s Off -> 0.9s On: UART TX Output 0.3s Off -> 0.3s On: UART RX Receive On: UART bidirection. |
| Power | Power LED | Power | O | On: Power input OK Off: Power input NG. |
| Link | Server connection LED | Link | O | On: netp Socket connection OK. Off: no netp Socket connection. |

<Notes>

I — Input; O — Output;  I/O: Digital I/O; Power—Power Supply

  - RS485 Interface

RS485 use two wire links, A(DATA+), B(DATA-). Connect A(+) to A(+), B(-) to B(-) for communication. Suggest to connect GND together when interference is very severe.

The RS485 interface support maximum 32 RS485 device. The cable maximum length is 1200 meters. Need to add 120Ohm terminal resistor for over 300 meters.

  - RJ45 Interface

Ethernet port is 10M/100M adaptive, support AUTO MDI/MDIX which means it support direct connecting to PC with Ethernet cable.

RJ45 Pin Defination

RJ45 Interface

| Pin Number | Name | Description |
| --- | --- | --- |
| 1 | TX+ | Transfer Data+ |
| 2 | TX- | Transfer Data- |
| 3 | RX+ | Receive Data+ |
| 4 | PHY-VCC | Transformer Tap Voltage |
| 5 | PHY-VCC | Transformer Tap Voltage |
| 6 | RX- | Receive Data- |
| 7 | N.C. | None Connect |
| 8 | N.C. | None Connect |

  - Mechanical Size

The dimensions of Protoss-PE11 are defined as following picture (mm):

Protoss-PE11 Mechanical Dimension

  - Product Installation

Product Installation

  - Order Information

Base on customer detailed requirement, Protoss-PE11 provide different configuration version, Details as below:

Protoss-PE11 Product Order Information

- Function DESCRIPTION

Refer to “IOT_Device_Series_Software_Funtion” document for more detailed function.

- Appendix A: Contact Information

------------------------------------------------------------------------------------------------------------

Address:  Room 1002,Building 1,No.3000,Longdong Avenue,Pudong New 					Area,Shanghai,China,201203

Web:      www.iotworkshop.com or www.hi-flying.com

Contact:

Sales: sales@iotworkshop.com

Support: support@iotworkshop.com

Service: service@iotworkshop.com

Business: business@iotworkshop.com

----------------------------------------------------------------------------  -------------------------------

For more information about IOTworkshop modules, applications, and solutions, please visit our web site www.iotworkshop.com

<END OF DOCUMENT>